	Weekly Scaffold Checklist
	


[image: image1.jpg]




	Project:- 
	Date:

	Contractor: 
	Location:

	Work description:
	Height of scaffold:

	Type of scaffold:
	Erected by: 


	SN
	Check
	Yes
	No
	NA

	A
	General
	
	
	

	1.1
	Is the scaffold plumb and level?
	
	
	

	1.2
	Erected by competent/ trained scaffolders?
	
	
	

	1.3
	Is scaffold properly designed and erected, with its intended use in mind?
	
	
	

	1.4
	Were the scaffold has been designed and constructed for loading with materials and are evenly distributed?
	
	
	

	1.5
	Has been erected on a leveled surface?
	
	
	

	1.6
	Does the height of the scaffold (tower) exceed 4 times of the least base dimension?
	
	
	

	1.7
	Are all uprights, provided with base plates and timber sole plates (1” thick) where necessary or prevented in some other way from slipping or sinking?
	
	
	

	1.8
	Has the ground beneath timber sole plates been properly leveled or compacted?
	
	
	

	1.9
	Have any uprights, ledgers, braces, clamps been removed?
	
	
	

	1.10
	Are back ties provided at every 6m vertically and 8m horizontally?
	
	
	

	1.11
	If any ties have been removed since the scaffold was erected, have additional ties been provided to replace them?
	
	
	

	B
	Working platform
	
	
	

	2.1
	Is fully and closely boarded, tied with MS gratings or 2” thick planks?
	
	
	

	2.2
	Is at least 0.7m wide if used only as a footing or 0.7 m in addition to existing, if used to store materials?
	
	
	

	2.3
	Are guardrails (1.2 m), knee rails (0.5 m) and toe boards (0.15 m) provided, where there is an exposure of fall of 2.0 m and more?
	
	
	

	2.4
	Are ladders clamped at a minimum of two points and extend a minimum of 1.2 m above the level of the platform it accesses?
	
	
	

	2.5
	Mid landing platforms provided for climbing scaffold at every 6m?
	
	
	

	2.6
	Constructed as close to the structure or building as possible with adequate back ties?
	
	
	

	SN
	Check
	Yes
	No
	NA

	2.7
	Are kept clean and free from grease, oil, rubble, debris or rubbish to ensure safe movement for personnel performing the works?
	
	
	

	C
	Mobile Tower scaffolds
	
	
	

	4.1
	All components used are free from damages?
	
	
	

	4.2
	Is height of working platform is 3.5 times the minimum base dimension if used internally and 3.0 times if used externally?
	
	
	

	4.3
	Wheels fitted at base with a locking device?
	
	
	

	4.4
	Stabilizers/ outriggers provided?
	
	
	

	4.5
	Access provided by attaching a ladder to its narrow side?
	
	
	

	D
	Housekeeping & other precautions?
	
	
	

	5.1
	Are there effective barriers or warning notices to stop people using an incomplete scaffold, e.g. one that is not fully boarded?
	
	
	

	5.2
	Are scaffold materials stored safely?
	
	
	

	5.3
	Are all walkways level and free from obstructions such as stored material and wastes?
	
	
	

	5.4
	Are there adequate barriers or other edge protection to stop falls from open edges of buildings, gangways, etc)?
	
	
	

	5.5
	Are holes and openings securely fenced off or provided with fixed, clearly marked covers?
	
	
	

	5.6
	Is the scaffold re-examined after rain or bad weather?
	
	
	

	Remarks/ Observations:


Name of the Inspector




Name of the engineer using Scaffold

Signature






Signature

Date






Date






1 of 1

